KWAME NKRUMAH UNIVERSITY OF SCIENCE AND TECHNOLOGY

(KNUST), KUMASI

DEPARTMENT OF COMPUTER SCIENCE

FACULTY OF PHYSICAL AND COMPUTATIONAL SCIENCES

FINAL YEAR PROJECT REPORT

IoT & ML-Based Predictive Car Engine Health Monitoring System

SUBMITTED BY

Iddrisu Faadila Wumpini

Lartey-Mensah Emmanuel Tetteh

Group 11

SUPERVISOR

Dr. R.O.M Gyening

A Project Report Submitted to the Department of Computer Science in Partial Fulfilment of the Requirements for the Award of the

Bachelor of Science Degree in Computer Science

ACADEMIC YEAR: 2025/2026

# CHAPTER ONE

# INTRODUCTION

## 1.1 Problem Statement

Car owners and fleet operators across Ghana and wider sub-Saharan Africa face a massive financial burden from sudden engine failure. Annual figures from the Ghana Road Safety Authority (GRSA) show that mechanical breakdown causes a large percentage of road accidents. Tragically, most of these trace back to engine faults that went completely unnoticed. Even with these risks, vehicle maintenance in Ghana is still almost entirely reactive. Drivers simply do not visit a fitter until a parts failure happens or the car stops completely.

Most modern cars use standard On-Board Diagnostic (OBD-II) systems. However, these tools are built to log fault codes only after a critical threshold trips its Engine Control Unit (ECU). They cannot spot issues such as minor mechanical wear early, subtle changes in vibration from an engine misfire, bad bearings, or loose mounts. OBD-II setups simply lack active, real-time tracking and early warning systems.

Because small transport operators and regular drivers cannot afford advanced diagnostic tools, early warning signals like minor temperature spikes or shifting vibration frequencies pass unnoticed. They eventually snowball into catastrophic mechanical breakdowns. This reality leaves vehicle owners facing ruinous repair bills, lost income from vehicle downtime, and hazardous driving conditions. Resolving this issue requires a cheap, IoT-driven system that watches critical engine metrics and uses machine learning to catch operational anomalies early.

## 1.2 Aim of the Project

This project builds and tests a less expensive alternative for predictive car engine maintenance. The setup tracks real-time engine vibration and surface temperature continuously to spot hidden operational anomalies. By catching these fault signatures early, the system sends immediate alerts to drivers or mechanics before catastrophic engine damage occurs.

## 1.3 Objectives of the Project

To achieve this aim, the study focuses on the following specific objectives:

- Review existing research: Analyse literature on vehicle diagnostics, IoT sensor networks, and unsupervised machine learning models used for spotting anomalies.

- Build the hardware prototype: Assemble a low-cost tracking unit using an ESP32 microcontroller, an MPU6050 accelerometer, and a DS18B20 digital temperature sensor to capture continuous engine data.

- Create the data preprocessing pipeline: Develop a software pipeline that cleans, normalizes, and extracts statistical features from raw vibration and temperature data streams.

- Train the machine learning model: Deploy an unsupervised Isolation Forest algorithm capable of separating normal engine baselines from developing mechanical faults.

- Deploy a secure dashboard: Design a web interface with secure user authentication to display real-time sensor metrics and trigger instant anomaly alerts.

- Test and evaluate the architecture: Assess the complete system by measuring its anomaly detection accuracy, response latency, and overall stability under simulated fault conditions.

## 1.4 Justification of the Project

Both technical and socio-economic realities in Ghana justify the development of this system. Ghana's transport sector relies heavily on private cars, taxis, ride-hailing services like Bolt, and commercial minibuses (trotros). Tight operating budgets mean most local drivers skip proactive maintenance, relying instead on emergency fixes because they cannot access or afford standard diagnostic tools. Merging low-cost IoT networks with machine learning completely changes this dynamic. The system's core unit — the ESP32 microcontroller — costs under GHS 100 on the local market. This low price point ensures the final prototype is genuinely affordable for everyday Ghanaian drivers and small-scale transport operators.

Academically, this study expands research into intelligent transport systems, edge computing, and applied machine learning within developing economies. Running an unsupervised Isolation Forest model in a resource-constrained environment proves that predictive maintenance does not require expensive, cloud-heavy infrastructure.

## 1.5 Motivation for Undertaking the Project

This project addresses the frequent, preventable engine breakdowns on Ghanaian roads that cause traffic congestion and financial hardship for vehicle owners. By employing affordable IoT hardware and open-source machine learning, the system offers a practical solution to detect early-stage mechanical faults while applying core Computer Science skills. The project aims to reduce road accidents by providing timely diagnostics and preventing sudden vehicle failures.

## 1.6 Scope of the Project

This project builds a functional prototype for an IoT-based engine health monitoring system. The specific scope of the implementation includes:

- Monitored parameters: The system tracks two primary health metrics — engine vibration across the X, Y, and Z axes using an MPU6050 accelerometer, and engine surface temperature via a DS18B20 sensor.

- Hardware framework: Data acquisition and wireless transmission run on an ESP32 microcontroller, which sends sensor data over Bluetooth Low Energy (BLE) directly to a connected laptop or phone.

- Machine learning model: Anomaly detection relies on an unsupervised Isolation Forest algorithm. This model learns exclusively from data representing normal engine operation.

- Validation method: System performance and detection accuracy are verified using the AI4I 2020 Predictive Maintenance Dataset under simulated fault conditions.

- User interface: A secure web dashboard features full user authentication — supporting both standard email/password logins and Google OAuth — to display real-time sensor streams and live anomaly alerts.

- System boundaries: This prototype excludes direct OBD-II port integration, GPS tracking, fuel monitoring, and native mobile apps. These elements remain open for future development.

## 1.7 Project Limitations

- Simulated training data: Using the AI4I 2020 simulated dataset for model training means the algorithm might not catch every unpredictable variation found in a real-world engine failure.

- Limited engine testing: Because testing was done on a single engine setup, different engine types or varied mounting positions could alter the vibration baseline and impact overall accuracy.

- Bluetooth range limitation: The prototype requires the receiving laptop or phone to remain within approximately 10 metres of the ESP32 sensor unit. Monitoring is interrupted if the device moves out of Bluetooth range.

- Single device connection: Standard Bluetooth Low Energy allows only one central device (laptop or phone) to connect to the ESP32 at a time, limiting simultaneous multi-user access.

- Mounting sensitivity: The accelerometer must be positioned correctly on the engine block. Any slight shift in physical placement or mounting stability can easily skew the vibration readings.

- Hardware processing limits: The ESP32 lacks the processing power to run heavy on-device inference. Consequently, all machine learning computations must be handled by an external backend device.

## 1.8 Beneficiaries of the Project

### Primary Beneficiaries

- Individual Car Owners: Real-time anomaly alerts enable preventive action before costly engine damage occurs.

- Ride-Hailing and Taxi Operators: Uber and Bolt drivers in Ghana benefit from reduced unplanned downtime and improved vehicle reliability.

### Secondary Beneficiaries

- Automotive Workshops and Mechanics: Access to anomaly history helps mechanics diagnose faults more accurately.

- Fleet Managers and Transport Companies: The system can scale to monitor multiple vehicles simultaneously.

- Academic and Research Community: This project provides a practical case study for applying IoT and machine learning to vehicle health monitoring in Ghana.

## 1.9 Academic and Practical Relevance of the Project

This study expands existing research by providing a solid example of unsupervised machine learning running on real-time IoT time-series data. Specifically, it bridges a significant gap in academic literature regarding how predictive maintenance tools can be built for developing economies. Outside the academic scope, the project delivers a functional, cost-effective prototype that shows AI-powered vehicle diagnostics can actually scale for the mass market. Using purely open-source tools means other developers can easily copy, scale, or commercialize the entire architecture without facing heavy upfront software costs.

## 1.10 Project Activity Planning and Schedule

Table 1.1: Project Activity Schedule

| Phase | Activity | Weeks |
| --- | --- | --- |
| Phase 1 | Literature Review & Planning — review of related works, requirements finalization, hardware procurement. | Weeks 1–2 |
| Phase 2 | Hardware Setup & Data Collection — ESP32, MPU6050, DS18B20 prototype assembly and sensor testing. | Weeks 3–5 |
| Phase 3 | Data Preprocessing & Feature Engineering — Butterworth filtering, normalization, windowing. | Weeks 6–7 |
| Phase 4 | Machine Learning Model Development — Isolation Forest training, evaluation, threshold tuning. | Weeks 8–10 |
| Phase 5 | Dashboard, Auth & System Integration — Streamlit, Supabase Auth, Google OAuth, BLE integration. | Weeks 11–12 |
| Phase 6 | Evaluation, Documentation & Dissertation Write-up — final testing and report completion. | Weeks 13–16 |

## 1.11 Structure of the Report

- Chapter 2 — Review of Related Works: Critical review of existing vehicle health monitoring systems and machine learning anomaly detection techniques.

- Chapter 3 — Methodology: Requirements specification, UML diagrams, software development model, UI wireframes, and database schema.

- Chapter 4 — Implementation and Results: Physical implementation, code snippets, screenshots, and testing results.

- Chapter 5 — Findings and Conclusion: Findings, conclusions, limitations, lessons learned, and recommendations.

## 1.12 Project Deliverables

- Hardware prototype: A fully wired monitoring unit combining the ESP32 microcontroller with an MPU6050 accelerometer and a DS18B20 temperature sensor.

- Python software pipeline: A custom script repository built to handle raw data cleaning, normalization, and time-series feature extraction.

- Trained ML model: An optimized unsupervised Isolation Forest algorithm calibrated to identify abnormal engine behavior from streaming data.

- Secure web interface: A front-end web portal featuring email/password and Google OAuth authentication, hosted on GitHub Pages.

- Live data dashboard: A real-time Streamlit dashboard deployed on Streamlit Cloud to visualize live sensor metrics instantly.

- Final dissertation: A comprehensive thesis document detailing the engineering design, system implementation, experimental testing, and key findings.

# CHAPTER TWO

# REVIEW OF RELATED WORKS

## 2.1 Chapter Overview

This chapter evaluates existing research and systems, analysing modern vehicle diagnostics, IoT sensor configurations, and machine learning models for anomaly detection to ground the project within current engineering practices. It also details the proposed system's architecture, components, and benefits.

## 2.2 Processes of Existing Systems

### 2.2.1 On-Board Diagnostic (OBD-II) Systems

Today, the On-Board Diagnostic (OBD-II) standard remains the most deployed technology for vehicle health tracking. Mandated globally in almost all vehicles built after 1996, OBD-II tools tap into the Engine Control Unit (ECU) to observe a rigid set of factory parameters. If any metric crosses a pre-programmed limit, the ECU logs a Diagnostic Trouble Code (DTC) and illuminates the dashboard check-engine light (Gao, Cecati & Ding, 2015).

The main advantage of OBD-II is its absolute standardization across the automotive industry, meaning it is universally supported and requires no extra hardware modifications. However, the architecture is inherently reactive. Because it only triggers an alert after a failure condition actually occurs, subtle mechanical wear — like early-stage bearing degradation or loose mounts — slips by completely unnoticed. Furthermore, reading these internal codes requires dedicated diagnostic scanners that remain financially out of reach for the average Ghanaian driver.

### 2.2.2 IoT-Based Vibration Monitoring Systems

Zhang and Ji (2019) proposed an IoT-based remote engine health monitoring system using a MEMS accelerometer and Fast Fourier Transform (FFT) analysis transmitted via Wi-Fi using the HTTP protocol. Their work demonstrated the feasibility of using low-cost accelerometers for engine monitoring, yet the system's reliance on static frequency thresholds for anomaly detection made it prone to false positives under varying engine loads.

### 2.2.3 LSTM-Based Anomaly Detection for Multi-Sensor Data

Malhotra et al. (2016) proposed an LSTM-based encoder-decoder model for multi-sensor time-series anomaly detection, which excels at identifying contextual anomalies by learning sequential dependencies from normal data. While effective, the model's high computational demands render it unsuitable for resource-constrained edge devices like the ESP32, and its validation was limited to a laboratory setting.

### 2.2.4 Industrial IoT Predictive Maintenance

Kanawaday and Sane (2017) utilized the Isolation Forest algorithm for unsupervised anomaly detection in IoT-based predictive maintenance, demonstrating effective detection with low computational overhead. Despite this efficiency, the approach relied on high-cost, industrial-grade hardware, making it financially impractical for individual vehicle owners in Ghana.

### 2.2.5 Commercial Fleet Monitoring Systems

Commercial platforms like Samsara and Geotab deliver extensive vehicle health tracking by combining GPS location data, direct OBD-II integration, and enterprise-grade cloud dashboards. While these professionally maintained architectures scale seamlessly for massive corporate fleets, they require high monthly subscription fees ranging from $25 to $70 USD per vehicle — financially impossible for individual vehicle owners and informal transport operators across Ghana.

## 2.3 Comparative Summary of Existing Systems

Table 2.1: Comparative Summary of Existing and Proposed Systems

| System / Study | Sensors | Protocol | ML / Detection | Key Limitation |
| --- | --- | --- | --- | --- |
| OBD-II (Gao et al., 2015) | ECU internal | CAN Bus | Rule-based fault codes | Reactive only; no prediction |
| Zhang & Ji (2019) | Accelerometer | Wi-Fi/HTTP | FFT + Static Threshold | No ML; false positives |
| Malhotra et al. (2016) | Multi-sensor | N/A (offline) | LSTM Encoder-Decoder | Lab-only; not IoT deployed |
| Kanawaday & Sane (2017) | Vib, Temp, Pressure | MQTT | Isolation Forest | Industrial hardware; costly |
| Commercial (Samsara) | GPS, OBD-II, Accel | 4G/LTE | Proprietary ML | $25–$70/month; unaffordable |
| PROPOSED SYSTEM | MPU6050, DS18B20 | Bluetooth BLE | Isolation Forest | Low-cost; no Wi-Fi needed; Ghana-context |

## 2.4 The Proposed System

This proposed IoT and machine learning setup directly solves the gaps left by traditional diagnostics. Made with the average Ghanaian car owner in mind, the prototype is affordable, processes metrics in real time, and requires zero specialized mechanical skills to install. While standard OBD-II setups look backward, this system captures active physical telemetry to find structural engine wear before factory fault codes ever trigger.

Rather than relying on a Wi-Fi network — which vehicles cannot always access as they move — the system uses Bluetooth Low Energy (BLE) to transmit sensor data directly from the ESP32 to a connected laptop or smartphone. This removes the dependency on a local Wi-Fi router and makes the system usable anywhere within Bluetooth range. Instead of using rigid, static sensor thresholds that trigger frequent false alarms, this architecture uses an unsupervised machine learning model that maps the engine's unique baseline and spots operational deviations dynamically.

## 2.5 Conceptual Design

The entire system relies on an integrated edge-to-device IoT architecture. At the edge, an ESP32 microcontroller interfaces directly with the hardware sensors, sampling the MPU6050 accelerometer at 50Hz and the DS18B20 temperature probe at 1Hz. This raw telemetry is transmitted over Bluetooth Low Energy (BLE) directly to a connected laptop running the Python inference engine. The backend processes all signal preprocessing and machine learning inference. Whenever the system flags an operational anomaly, it pushes the alert event to a cloud database and updates the web-based monitoring dashboard simultaneously. Dashboard access is restricted by a Supabase Auth gateway, which handles user sign-ups through mandatory verification emails as well as standard Google OAuth logins.

## 2.6 Architecture of the Proposed System

### Layer 1: Perception Layer

The physical hardware layer mounts the MPU6050 MEMS accelerometer and the DS18B20 digital temperature sensor directly onto the vehicle's engine block. The MPU6050 maps continuous 3-axis acceleration to track engine vibration, while the DS18B20 maintains precise tracking of surface temperature fluctuations.

### Layer 2: Network Layer

The ESP32 compiles sensor readings into JSON-formatted payloads and transmits them to a connected laptop via Bluetooth Low Energy (BLE). The ESP32 operates as a BLE peripheral, advertising itself as "EngineIQ_Sensor" and sending sensor notifications to the subscribed central device. This eliminates the need for a Wi-Fi router or MQTT broker entirely.

### Layer 3: Processing Layer

The Python inference engine running on the connected laptop receives BLE notifications and processes the raw telemetry in three distinct stages. First, it sanitizes the data stream using a digital Butterworth filter, normalizes the values, and splits the time-series data into discrete windows. Second, it extracts 22 statistical features — including mean, standard deviation, RMS, kurtosis, skewness, range, and peak-to-peak metrics. Finally, it runs machine learning inference on these features to generate real-time anomaly scores using the trained Isolation Forest model.

### Layer 4: Application Layer

The user interface features a live Streamlit web dashboard that charts real-time time-series plots, displays the active anomaly score indicator, and catalogs historical event logs. Users access this monitoring interface via the project landing page at https://faadila-wumpini.github.io/Engine_monitor, which requires full credentials authentication before unlocking the live sensor streams.

## 2.7 Component Descriptions

### 2.7.1 ESP32 Microcontroller

The ESP32 is a low-cost, low-power system-on-chip (SoC) featuring a dual-core Xtensa LX6 processor at up to 240 MHz, 520 KB SRAM, and integrated Bluetooth (Classic and BLE). In the proposed system it serves as the central edge device for sensor interfacing, data acquisition, and wireless BLE transmission. The ESP32 advertises as a BLE peripheral named "EngineIQ_Sensor" and sends sensor readings as BLE characteristic notifications to the connected laptop.

### 2.7.2 MPU6050 MEMS Accelerometer

The MPU6050 is a six-axis MEMS IMU communicating via I2C. Only the accelerometer functionality is used, measuring linear acceleration along X, Y, and Z axes. Raw 16-bit readings are converted to g-force by dividing by 16,384 (sensitivity at ±2g range).

### 2.7.3 DS18B20 Digital Temperature Sensor

The DS18B20 provides 9 to 12-bit temperature measurements with ±0.5°C accuracy over -10°C to +85°C, communicated via the Dallas 1-Wire protocol over a single GPIO pin. A 4.7kΩ pull-up resistor is connected between the data line and 3.3V supply.

### 2.7.4 Bluetooth Low Energy (BLE) Communication

Bluetooth Low Energy (BLE) is the wireless communication protocol used to transfer sensor data from the ESP32 to the processing laptop. BLE is specifically designed for low-power, short-range data transmission, making it ideal for IoT sensor devices. The ESP32 acts as a BLE peripheral (server) and exposes a custom GATT service with a sensor data characteristic. The Python inference engine on the laptop acts as a BLE central (client) using the Bleak library to scan for, connect to, and subscribe to BLE notifications from the ESP32. Data is packaged as JSON strings and transmitted at the 50Hz vibration sampling rate. BLE eliminates the need for a Wi-Fi router, MQTT broker, or internet connection for local data acquisition.

### 2.7.5 Machine Learning Model — Isolation Forest

The Isolation Forest (Liu, Ting & Zhou, 2008) is an unsupervised anomaly detection algorithm that isolates observations by randomly selecting features and split values. Anomalous data points require fewer tree splits to isolate, yielding lower anomaly scores. The algorithm is computationally efficient, distribution-free, and trained exclusively on normal engine data.

### 2.7.6 Supabase Cloud Database and Authentication

Supabase is an open-source Firebase alternative built on PostgreSQL. It provides a managed cloud database with auto-generated REST APIs and a complete user authentication service. Supabase Auth supports email/password authentication with email confirmation, Google OAuth 2.0, and JWT-based session management. In the proposed system, Supabase Auth secures the project website — users must register and confirm their email before accessing the monitoring dashboard. The profiles table stores user full names and emails to support name-based login.

### 2.7.7 Streamlit Web Dashboard

Streamlit is an open-source Python library for building interactive data web applications. In the proposed system it powers the user-facing monitoring dashboard, displaying real-time time-series charts, anomaly score gauges, and an alert history table populated from Supabase.

## 2.8 Proposed System Features

- Real-Time Monitoring: Continuous acquisition and display of vibration data at 50Hz and temperature data at 1Hz, transmitted over Bluetooth with sub-second latency.

- Unsupervised Anomaly Detection: ML model trained solely on normal engine data, requiring no labelled fault data.

- Multi-Axis Vibration Analysis: Vibration capture across X, Y, and Z axes enabling detection of directional fault signatures.

- Bluetooth Wireless Transmission: Sensor data is transmitted via BLE, eliminating the need for a Wi-Fi network or internet connection for data acquisition.

- Historical Anomaly Logging: Persistent storage of all detected anomaly events via Supabase cloud database.

- User Authentication: The project website is secured with Supabase Auth. Users must register and confirm their email before accessing the dashboard. Google OAuth is provided as an alternative sign-in method.

- Password Security: Client-side password validation enforces a minimum of 8 characters and at least one special character, with a live strength indicator and confirm-password match indicator.

- Low-Cost and Accessible: Estimated total hardware cost below GHS 400.

- Severity Classification: Anomalies are classified into Low, Medium, and High severity levels.

- Dual Operation Mode: The inference engine can run in live Bluetooth mode when the ESP32 is connected, or automatically fall back to simulation mode using the AI4I dataset when hardware is unavailable.

## 2.9 Development Tools and Environment

### 2.9.1 Hardware Tools

The hardware development environment consists of the ESP32 development board (ESP-WROOM-32), MPU6050 accelerometer module, DS18B20 temperature sensor, a 4.7kΩ pull-up resistor, jumper wires, a breadboard, and the Arduino IDE (version 2.x).

### 2.9.2 Software Tools

Python (version 3.13) is the primary programming language for the data processing pipeline, ML model, and dashboard. Key libraries include: Pandas and NumPy for data manipulation; SciPy for signal processing; Scikit-learn for Isolation Forest; Bleak for Bluetooth BLE communication; Supabase-py for cloud database integration; and Streamlit for the web dashboard.

### 2.9.3 Development Environment

Development is conducted on Windows 11 with Visual Studio Code as the primary code editor. Git and GitHub are used for version control and collaboration. The project website is deployed via GitHub Pages and the dashboard via Streamlit Cloud.

## 2.10 Benefits of the Proposed System

- Proactive Fault Detection: Detecting anomalies before they escalate enables preventive maintenance, significantly reducing repair costs.

- Cost Effectiveness: At an estimated hardware cost below GHS 400, the system is accessible to individual car owners in Ghana.

- No Wi-Fi Required: Bluetooth BLE transmission works anywhere within 10 metres of the sensor unit, without needing a router or internet connection for data acquisition.

- Secure Access: User authentication ensures only registered users can access the monitoring dashboard, protecting data privacy.

- Ease of Use: The dashboard provides a simple, intuitive interface requiring no technical expertise.

- Scalability: Multiple ESP32 sensor units can be paired to different laptops or phones for multi-vehicle monitoring.

## 2.11 Chapter Summary

This chapter provided a comprehensive review of existing vehicle health monitoring systems and related works. Critical gaps were identified — the reactive nature of OBD-II diagnostics, limitations of threshold-based IoT systems, and prohibitive cost of commercial platforms. The proposed system was presented as a solution through low-cost IoT hardware, Bluetooth Low Energy communication, unsupervised machine learning, and a secure user-authenticated web interface. Chapter 3 will present the methodology adopted for system development.

# CHAPTER THREE

# METHODOLOGY

## 3.1 Chapter Overview

This chapter presents the methodology adopted for the design and development of the system. It begins with requirements specification, UML diagrams, non-functional requirements, security concepts, the software development model, and logical design considerations including UI wireframes and database schema.

## 3.2 Requirement Specification

### 3.2.1 Stakeholders of the System

- New User: A person who registers a new account on the project website to gain access to the monitoring dashboard.

- Registered Vehicle Owner / Driver (Primary User): The authenticated end user who monitors engine health via the dashboard and receives anomaly alerts.

- System Administrator: A technical user responsible for model training, threshold configuration, and system maintenance.

- Processing Server / ML Engine (System Actor): The backend component that receives BLE sensor data, executes preprocessing, and runs anomaly detection inference.

- ESP32 Sensor Node (System Actor): The embedded hardware device that collects raw sensor data and transmits it via Bluetooth Low Energy to the connected processing device.

### 3.2.2 Requirement Gathering Process

Requirements were gathered through: literature review of related vehicle health monitoring and predictive maintenance systems; stakeholder analysis of individual car owners and fleet operators in Ghana; review of the project proposal document; consultation with the project supervisor; and iterative refinement during Agile development sprints.

## 3.3 Functional Requirements

Table 3.1: Functional Requirements

| ID | Requirement | Priority |
| --- | --- | --- |
| FR-01 | The system shall continuously read vibration data from the MPU6050 accelerometer at a minimum rate of 50Hz. | High |
| FR-02 | The system shall continuously read surface temperature from the DS18B20 sensor at a minimum rate of 1Hz. | High |
| FR-03 | The ESP32 shall format sensor readings as JSON payloads and transmit them to the connected device via Bluetooth Low Energy (BLE) notifications in real-time. | High |
| FR-04 | The Python inference engine shall receive BLE sensor data with a connection latency under 100ms. | High |
| FR-05 | The preprocessing pipeline shall apply a Butterworth low-pass filter to remove noise from vibration data. | High |
| FR-06 | The system shall extract at least seven statistical features per sensor axis per time window. | High |
| FR-07 | The ML model shall classify each feature vector as normal or anomalous using an anomaly score threshold. | High |
| FR-08 | The system shall log all detected anomaly events to a Supabase database with timestamp and severity. | High |
| FR-09 | The dashboard shall display real-time vibration and temperature charts refreshed at ~1-second intervals. | High |
| FR-10 | The dashboard shall display a visual anomaly alert banner within 2 seconds of a fault event being detected. | High |
| FR-11 | The dashboard shall display a historical log of the 50 most recent anomaly events from the database. | Medium |
| FR-12 | An administrator panel shall allow authorised users to log in and adjust the anomaly score threshold. | Medium |
| FR-13 | The system shall classify anomaly severity into three levels: Low, Medium, and High. | Medium |
| FR-14 | The website shall require users to register an account with full name, email address, and password before accessing the monitoring dashboard. | High |
| FR-15 | The system shall send a confirmation email to newly registered users. The confirmation link shall expire after one hour. | High |
| FR-16 | The website shall allow users to sign in using either their registered email address or their full name combined with their password. | High |
| FR-17 | The system shall provide a 'Continue with Google' OAuth option on both Sign In and Create Account screens. | High |
| FR-18 | The signup form shall enforce a minimum password length of 8 characters including at least one special character, with a live strength indicator and confirm-password match indicator. | High |
| FR-19 | If a confirmation email is not received, the user shall be able to request a resend after a 60-second countdown. | Medium |
| FR-20 | The inference engine shall automatically detect whether the ESP32 is reachable via Bluetooth and switch to simulation mode using the AI4I dataset if no device is found. | High |

## 3.4 UML Diagrams

### 3.4.1 Use Case Diagram — Front-End (Driver Dashboard & Authentication)

The front-end use case diagram captures interactions between two actor types and the web-based system. The New User actor interacts with four authentication use cases: Register Account, Confirm Email Address, Resend Confirmation Email, and Sign In with Google. The Registered Vehicle Owner actor interacts with: Sign In (email or name), View Real-Time Vibration Chart, View Real-Time Temperature Chart, Receive Anomaly Alert Notification, View Anomaly Severity Status, View Anomaly History Log, and Sign Out.

[ Figure 3.1 — Use Case Diagram: Driver Dashboard (Front-End) ]

### 3.4.2 Use Case Diagram — Back-End (System Administrator)

The back-end use case diagram captures interactions between the System Administrator and the backend processing components. The administrator can log in, configure the anomaly score threshold, trigger ML model retraining, view system status indicators, view the full anomaly log, and export the anomaly log as CSV.

[ Figure 3.2 — Use Case Diagram: System Administrator (Back-End) ]

### 3.4.3 Use Case Descriptions

Table 3.2: Use Case Descriptions

| Use Case ID | Use Case Name | Actor | Description |
| --- | --- | --- | --- |
| UC-01 | View Real-Time Vibration Chart | Vehicle Owner | The driver views a continuously updating line chart showing MPU6050 acceleration readings on X, Y, and Z axes over the last 100 data points. The chart auto-refreshes every second. |
| UC-02 | View Real-Time Temperature Chart | Vehicle Owner | The driver views a continuously updating chart showing DS18B20 surface temperature over time, enabling detection of overheating conditions. |
| UC-03 | Receive Anomaly Alert | Vehicle Owner | When the ML model detects an anomaly, the dashboard displays a prominent red alert banner with the anomaly score and severity level. |
| UC-04 | View Anomaly Severity Status | Vehicle Owner | A colour-coded status indicator (green: Normal, yellow: Low, orange: Medium, red: High) is displayed on the dashboard at all times. |
| UC-05 | View Anomaly History Log | Vehicle Owner | The driver views a table of the 50 most recent anomaly events retrieved from the Supabase database, including timestamp, severity, and anomaly score. |
| UC-06 | Filter Anomaly History by Date | Vehicle Owner | The driver selects a date range to filter displayed anomaly records to a specific time window. |
| UC-07 | Log In to Admin Panel | Administrator | The administrator enters credentials. Supabase Auth validates them and grants access to administration controls. Invalid credentials result in an error message. |
| UC-08 | Configure Anomaly Threshold | Administrator | The administrator adjusts the anomaly score threshold using a slider. The new threshold is immediately applied to subsequent inference cycles. |
| UC-09 | Trigger ML Model Retraining | Administrator | The administrator initiates a model retraining cycle. The new model and scaler are saved to disk and loaded into the active inference engine. |
| UC-10 | Register Account | New User | A new user fills the Create Account form with full name, email, password (min. 8 chars with special character), and confirm password. Supabase Auth creates the account and sends a confirmation email with a 60-second resend countdown. |
| UC-11 | Confirm Email Address | New User | The user clicks the confirmation link in their email. The Supabase SDK detects the token, activates the account, and the user is automatically signed in. |
| UC-12 | Sign In with Email or Name | Registered User | The user enters either their email address or full name, plus their password. If a name is entered, the system queries the profiles table to retrieve the associated email. Supabase Auth validates the credentials. |
| UC-13 | Sign In with Google | New / Registered User | The user clicks Continue with Google. After approval, Supabase creates or retrieves the account, issues a JWT session, and redirects the user to the dashboard. |
| UC-14 | Resend Confirmation Email | New User | After the 60-second countdown expires, the user clicks Resend. Supabase sends a new confirmation email and the countdown restarts. |
| UC-15 | Sign Out | Authenticated User | The user clicks Sign Out in the navigation bar. Supabase clears the JWT session and the authentication screen is displayed. |

### 3.4.4 Activity Diagram — Sensor Data Flow

The activity diagram describes the end-to-end data flow: the ESP32 reads sensors and transmits JSON payloads via BLE notifications; the Python inference engine on the laptop receives the BLE data and appends it to a rolling buffer; when the buffer reaches 50 samples, the Butterworth filter is applied; statistical features are extracted; Isolation Forest inference runs; if the score is below threshold, the anomaly is logged to Supabase and an alert is displayed on the dashboard.

[ Figure 3.3 — Activity Diagram: End-to-End Sensor Data Flow ]

### 3.4.5 Sequence Diagram — Anomaly Detection Flow

The sequence diagram illustrates temporal interactions during an anomaly detection event: ESP32 sends BLE notification to inference engine; inference engine receives payload, preprocesses it, and scores the feature vector; if anomaly detected, inserts record to Supabase; dashboard auto-refresh retrieves the latest record and displays the alert.

[ Figure 3.4 — Sequence Diagram: Anomaly Detection Flow ]

### 3.4.6 Class Diagram

The principal software classes are: DataPreprocessor (butterworth_filter, extract_features, process_dataframe, normalise methods), AnomalyDetector (train, predict, get_severity, save_model, load_model), DatabaseLogger (log_anomaly, get_history), and InferenceEngine (run_simulation, run_bluetooth, auto_detect, score_window) which composes all three. StreamlitDashboard uses DatabaseLogger and AnomalyDetector.

[ Figure 3.5 — Class Diagram: Principal Software Classes and Relationships ]

### 3.4.7 Authentication Flow Diagram

The authentication flow diagram illustrates how users access the system. On visiting the site, an active session check is performed. If a valid JWT session exists, the user is taken directly to the dashboard. If not, the authentication screen is displayed. The user may choose email/password (new user signs up and confirms email; returning user signs in by email or full name) or Google OAuth. All pathways result in a JWT session being established, after which the dashboard is displayed.

[ Figure 3.6 — Authentication Flow Diagram ]

## 3.5 Non-Functional Requirements

Table 3.3: Non-Functional Requirements

| ID | Category | Requirement | Justification |
| --- | --- | --- | --- |
| NFR-01 | Performance | End-to-end latency from sensor anomaly to dashboard alert shall not exceed 2 seconds. | Timely alerts are critical for the system's utility as an early warning tool. |
| NFR-02 | Reliability | The system shall achieve a true positive rate of at least 85% and false positive rate of no more than 5%. | Low false positives prevent alert fatigue; high true positives ensure faults are not missed. |
| NFR-03 | Availability | The dashboard shall be accessible from any modern web browser without software installation. | Broad compatibility ensures usability by non-technical car owners on any device. |
| NFR-04 | Scalability | The BLE architecture shall support pairing of additional ESP32 sensor units to separate processing devices for multi-vehicle monitoring. | Enables future scaling to fleet monitoring deployments. |
| NFR-05 | Usability | A non-technical user shall interpret engine health status without prior training. | The primary target user is not technically trained; the interface must be intuitive. |
| NFR-06 | Cost | Total hardware cost shall not exceed GHS 400. | Affordability is a primary design constraint for deployment in Ghana. |
| NFR-07 | Security | Unauthenticated users shall not be able to access any monitoring dashboard content. | Protecting user data and system integrity requires access control. |
| NFR-08 | Maintainability | All Python source code shall follow PEP-8 standards and include inline documentation. | Maintainable code supports future extension and handover. |

## 3.6 Security Concepts

The system addresses security requirements across six areas:

- Administrator Authentication: The admin panel is protected by Supabase Authentication with JWT-based session management. Unauthenticated users are denied access to administrative controls.

- Bluetooth Pairing Security: BLE communication between the ESP32 and the processing laptop is restricted by a unique 128-bit service UUID. Only a device that knows the exact service UUID and characteristic UUID can subscribe to sensor data, preventing unauthorised devices from intercepting the data stream.

- Database Row-Level Security: Supabase Row-Level Security (RLS) policies are configured on the anomaly_logs table. The profiles table has policies restricting read access and allowing users to only insert and update their own records.

- Data Transmission to Cloud: While BLE handles local sensor-to-laptop communication, data sent from the laptop to Supabase travels over HTTPS, which is encrypted by TLS by default.

- User Authentication — Email/Password: The website enforces user registration before access. Passwords are validated client-side for a minimum of 8 characters and at least one special character. Supabase Auth handles server-side password hashing using bcrypt. Email confirmation is required before an account can be used.

- User Authentication — Google OAuth: The system integrates Google OAuth 2.0. Google verifies the user's identity and issues an authorisation code to the Supabase callback endpoint. Supabase exchanges this for a JWT session token. No password is required or stored for Google-authenticated users.

## 3.7 Software Development Methodology

### 3.7.1 Overview of Software Process Models

- Waterfall Model: Linear sequential model suited to projects with fully defined requirements. Inflexible once development begins.

- Spiral Model: Combines iterative development with risk analysis. Appropriate for large high-risk projects but carries significant planning overhead.

- Prototype Model: Focuses on rapid construction of working prototypes. Effective for clarifying requirements but can lead to poor architecture.

- Agile Model: Iterative and incremental model organised around short development sprints. Each sprint produces a working, tested system increment.

### 3.7.2 Chosen Model: Agile

The Agile model was selected. Development is organised into six two-week sprints: Sprint 1 (literature review and hardware procurement), Sprint 2 (ESP32 BLE firmware and sensor integration), Sprint 3 (data preprocessing pipeline), Sprint 4 (ML model training and evaluation), Sprint 5 (dashboard, authentication, Supabase integration, and BLE inference engine), and Sprint 6 (end-to-end system testing and dissertation write-up).

### 3.7.3 Justification for Agile

- Adaptability to Hardware Constraints: IoT projects frequently encounter unforeseen integration challenges. Agile enabled rapid identification and resolution without requiring a full project restart.

- Alignment with Dual-Component Development: Agile allowed both hardware firmware (C++) and software (Python) development to proceed concurrently within the same sprint cycle.

- Continuous Integration and Testing: Agile's emphasis on a working increment at sprint end ensured hardware-software integration was tested continuously.

## 3.8 Project Design Considerations (Logical Designs)

### 3.8.1 UI Wireframes

Driver Dashboard Wireframe: The driver-facing dashboard presents a top-level status banner (NORMAL in green, ANOMALY in red), followed by real-time line charts for vibration (X, Y, Z axes) and temperature, an anomaly score indicator, a colour-coded severity badge, and an anomaly history table showing the last 50 events. The dashboard is accessible only after successful authentication.

[ Figure 3.7 — UI Wireframe: Driver Dashboard ]

Authentication Screen Wireframe: The authentication screen presents a Sign In tab (email or name identifier field, password field with eye toggle, Continue with Google button) and a Create Account tab (full name, email, password with strength bar, confirm password with match indicator, Continue with Google button). After successful signup, the form is replaced by a confirmation panel showing the destination email, a 1-hour expiry warning, and a resend button with 60-second countdown.

[ Figure 3.8 — UI Wireframe: Authentication Screen ]

Administrator Panel Wireframe: The administrator panel presents a login screen, then a sidebar with navigation items (System Overview, Threshold Config, Model Retraining, Anomaly Log, Export Data), system status indicators for BLE connection, Inference Engine, and Supabase, an anomaly threshold slider, a retrain button, and a full anomaly log table.

[ Figure 3.9 — UI Wireframe: Administrator Panel ]

### 3.8.2 Database Design — Entity-Relationship Diagram

The system uses two tables in the Supabase PostgreSQL database. The AnomalyLog entity records each detected anomaly event with attributes: id (UUID, PK), timestamp, acc_x_mean, acc_y_mean, acc_z_mean, temp_mean, anomaly_score, severity, and is_confirmed. The Profile entity stores user registration data with attributes: id (UUID, PK, references auth.users), email, and full_name.

[ Figure 3.10 — Entity-Relationship Diagram: Supabase Database Schema ]

### 3.8.3 Database Schema

Table 3.4: Database Schema — anomaly_logs Table

| Column | Data Type | Constraints | Description |
| --- | --- | --- | --- |
| id | UUID | PRIMARY KEY, auto-generated | Unique identifier for each anomaly record. |
| timestamp | TIMESTAMPTZ | NOT NULL | UTC timestamp of the detected anomaly event. |
| acc_x_mean | FLOAT8 | NOT NULL | Mean X-axis acceleration of the anomalous window (g). |
| acc_y_mean | FLOAT8 | NOT NULL | Mean Y-axis acceleration of the anomalous window (g). |
| acc_z_mean | FLOAT8 | NOT NULL | Mean Z-axis acceleration of the anomalous window (g). |
| temp_mean | FLOAT8 | NOT NULL | Mean surface temperature of the anomalous window (°C). |
| anomaly_score | FLOAT8 | NOT NULL | Isolation Forest anomaly score (lower = more anomalous). |
| severity | TEXT | NOT NULL | Classified severity: Low (> -0.52), Medium (> -0.55), High (≤ -0.55). |
| is_confirmed | BOOLEAN | DEFAULT false | Administrator flag confirming the anomaly as a true positive. |

Table 3.5: Database Schema — profiles Table

| Column | Data Type | Constraints | Description |
| --- | --- | --- | --- |
| id | UUID | PRIMARY KEY, references auth.users(id) CASCADE | Matches Supabase Auth user ID. Deleted when user account is deleted. |
| email | TEXT | NOT NULL | The user's registered email address. Used for name-based login lookup. |
| full_name | TEXT | NOT NULL | The user's full name as entered during registration. Used as an alternative login identifier. |

## 3.9 Developmental Tools

- Arduino IDE / PlatformIO: Used to write, compile, and flash C++ BLE firmware onto the ESP32. Libraries installed: MPU6050, DallasTemperature, OneWire, and the ESP32 BLE Arduino library (included with the ESP32 board package).

- Python 3.13: Primary language for all server-side components. Virtual environments manage package dependencies.

- Bleak Python Library: Used by the inference engine to scan for, connect to, and receive BLE notifications from the ESP32. Bleak is a cross-platform BLE library that works on Windows, macOS, and Linux using the native Bluetooth stack.

- Supabase: Used as the backend database and authentication service. Supabase Auth handles user registration, email confirmation, Google OAuth, and JWT session management. The supabase-py Python client inserts anomaly records and retrieves historical data for the dashboard.

- Streamlit: Used to build the interactive web dashboard. Deployed publicly via Streamlit Cloud at enginemonitor.streamlit.app.

- Git / GitHub: Used for version control. The project website is deployed via GitHub Pages at https://faadila-wumpini.github.io/Engine_monitor.

## 3.10 Chapter Summary

This chapter details the technical methodology, mapping twenty functional and eight non-functional requirements across five stakeholder types, including six specific authentication requirements (FR-14 to FR-19) and one new auto-detection requirement (FR-20). The communication protocol was updated from Wi-Fi/MQTT to Bluetooth Low Energy (BLE), reflecting the supervisor's recommendation to remove the Wi-Fi dependency and enable monitoring anywhere within Bluetooth range. The security architecture now utilises BLE UUID-based access control, Supabase RLS, JWT, HTTPS, and OAuth 2.0. The Agile development framework is maintained. Logical designs include an updated ERD, UI wireframes, and a Supabase database schema. Chapter 4 will present the physical implementation of these designs.

# REFERENCES

Espressif Systems. (2023). ESP32 Series Datasheet. Retrieved from https://www.espressif.com/sites/default/files/documentation/esp32_datasheet_en.pdf

Gao, Z., Cecati, C., & Ding, S. X. (2015). A survey of fault diagnosis and fault-tolerant techniques — Part I: Fault diagnosis with model-based and signal-based approaches. IEEE Transactions on Industrial Electronics, 62(6), 3757–3767.

InvenSense (TDK). (2013). MPU-6000 and MPU-6050 Product Specification Revision 3.4. Retrieved from https://invensense.tdk.com/wp-content/uploads/2015/02/MPU-6000-Datasheet1.pdf

Kanawaday, A., & Sane, A. (2017). Machine learning for predictive maintenance of industrial machines using IoT sensor data. IEEE International Conference on Computing, Communication, Control and Automation (ICCUBEA), Pune, India.

Liu, F. T., Ting, K. M., & Zhou, Z. H. (2008). Isolation Forest. Proceedings of the IEEE 8th International Conference on Data Mining (ICDM), Pisa, Italy, pp. 413–422.

Malhotra, P., Ramakrishnan, A., Anand, G., Vig, L., Agarwal, P., & Shroff, G. (2016). LSTM-based encoder-decoder for multi-sensor anomaly detection. arXiv preprint arXiv:1607.00148.

Maxim Integrated (Analog Devices). (2019). DS18B20 Programmable Resolution 1-Wire Digital Thermometer Datasheet. Retrieved from https://www.analog.com/media/en/technical-documentation/data-sheets/DS18B20.pdf

Scikit-learn Developers. (2023). sklearn.ensemble.IsolationForest. Retrieved from https://scikit-learn.org/stable/modules/generated/sklearn.ensemble.IsolationForest.html

Shimpi, P., Agrawal, S., & Kaul, A. (2021). Bluetooth Low Energy (BLE) based IoT systems: A survey. International Journal of Computer Applications, 174(12), 1–6.

Streamlit Inc. (2024). Streamlit Documentation. Retrieved from https://docs.streamlit.io

Supabase. (2024). Supabase Documentation. Retrieved from https://supabase.com/docs

Zhang, Y., & Ji, S. (2019). IoT-based remote monitoring system for engine health using vibration analysis. Journal of Physics: Conference Series, 1187(4).